Technical Data Sheet (TDS)

CR-580 DEGREASER

Exterior General Cleaning

Spray CR-580 Degreaser on the concrete and scrub with a broom then rinse with pressure washer.

Interior General Cleaning

Spray CR-580 Degreaser on the concrete and scrub using a floor machine (buffer).

Oil Spots

Saturate the affected area with CR-580 Degreaser and scrub with a stiff bristle brush, then pressure wash. Repeat as necessary to remove the oil on the surface. Note: On many oil spots a shadow of the oil spot will still remain even after several treatments. It is important to clean the oil spot until it no longer repels water.

Stripping Paint and Sealers

After using CR-590 Paint & Sealer Remover to remove paint, sealer, glue, or mastic, spray a liberal amount of CR-580 Degreaser on the floor and scrub with a floor machine (buffer) using a nylon bristle brush attachment, black pad or sanding screen. This process is imperative to remove residual oils and debris that may inhibit the ability of stain to react or penetrate.

Storage

Do not allow to freeze.

Keep away from children.

Safety

Do not breathe vapors. When using in confined or limited ventilation areas, use appropriate respirator to protect against breathing vapors. Avoid contact with skin; wear protective gloves and clothing. Always use eye protection such as goggles, face shield or safety glasses. Read Material Safety Data Sheet before using.

Caution

Eye and skin irritant

First Aid

Eye Contact: Immediately flush eyes with plenty of water for at least 15 minutes while holding eyelids open. Seek medical attention.

Skin Contact: Immediately remove contaminated clothing. Wipe excess from skin and wash with soap and water. Seek medical attention if irritation persists.

Inhalation: Remove person to fresh air and provide oxygen if breathing is difficult. Seek medical attention.

Warning

Do not mix with any other chemicals as hazardous gases may result.

Do not allow CR-580 Degreaser to dry on stained or sealed surfaces.

Product

CR-580 is high strength degreaser for removing grease, grime, and oily films.